<APT eMotion>Instruction

Official User Manual

1 Installation & Connection

- Unzip the APT eMotion customer.rar

- Double-click install.exe to launch the installer.

- Follow the wizard to select an installation path, then click Next.

- Wait for the installation to complete.

2 Connection & Startup

- Hardware Connection：

Connect your computer to the controller using a data cable (such as a USB adapter) and power on the controller normally.

- Software Launch：

Open the installed APT eMotion PC software.

- Establish Communication：

Select the correct serial port. Default baud rate: 57600.

- Click Open Serial Port to connect; Close Serial Port to disconnect.

（If the port is not detected immediately, click Refresh.）

- Special Notes: The monitoring serial port and online flashing port cannot be used simultaneously, or communication will fail.

3 Overview of the Host Machine Page Details

- Monitor Management

This page appears blank when the controller is disconnected

- The sold motor controller is pre-installed with standard firmware. You can connect it to the upper computer to view the system status in real time and modify parameters according to the manual or your actual requirements.

- Attention：Some parameters are fixed and non-adjustable on the upper computer, because the standard firmware is configured to be compatible with most vehicle models.If you have customized requirements, please contact our customer service team. We will conduct remote code rewriting and customized firmware development according to your needs.The customized program will then be written into the controller via online flashing software.

- Special Note: The upper computer serial port and the online flashing serial port cannot be used simultaneously; otherwise, the reading operation will fail.

3.1 Monitor

- System Operating States

- IDLE: Idle state

- STARTUP: P-mode active

- PARASET: Parameter setting mode

- RUN: Running

- STOP: Stopped

- FAULT: Fault condition

- Function Status Indicators

- The indicator lights up when the function is active:

- P-mode: Applies fixed DC current to lock the motor for anti-rollaway and temporary parking assistance.

- Forward: Converts battery DC to three-phase AC for forward motion (core driving function).

- Reverse: Reverses phase sequence for low-speed reverse assistance (safety-limited, short-distance use only).

- Anti-Theft: Locks the motor, cuts power, and triggers an alarm with an external alarm module.

- Cruise: Maintains constant speed/throttle without holding the grip. Not autonomous driving.

- Brake: Power cut + electronic brake (regeneration). Highest-priority safety function.

- Side Stand: Safety interlock via microswitch; prevents riding with the side stand down.

- Auto-Calibration: Automatic motor parameter learning (Hall sequence, pole pairs, back EMF).

- Before calibration: Lift the wheel so it rotates freely.

- Torque Mode: Throttle directly controls output torque (similar to a manual transmission).

- Charge Protection: Prevents vehicle movement during charging; part of the three-level charging safety system.

- Real-Time Operating Data

- Controller Temperature (°C): Temperature of internal power components; overheating triggers shutdown.

- Motor Temperature (°C): Motor winding temperature; triggers thermal protection.

- Supply Voltage (V): DC bus voltage from the battery; reference for all electrical parameters.

- Throttle Voltage (V): Analog signal from the throttle grip (0.8–4.2 V).

- Bus Current (A): Total battery discharge current.

- Motor Speed (RPM): Motor rotor revolutions per minute.

- Key Operating Parameters

- Current Parameters (FOC)

- Iq Reference (A): Target torque-producing current (primary FOC parameter).

- Iq Feedback (A): Actual measured torque current; should match reference within ±5%.

- Id Reference (A): Target field-weakening current (usually negative to increase top speed).

- Id Feedback (A): Actual field-weakening current.

- Is Reference (A): Total phase current limit (vector sum of Iq and Id).

- Is Feedback (A): Actual phase current; triggers overcurrent/short-circuit protection.

- Motor Parameters

- Position Status: Real-time rotor electrical angle (0–360°); foundation of FOC control.

- Speed Command: Target speed in speed mode.

- Other Parameters

- Field-Weakening Angle: Sets the ratio of Id to Iq; higher angle = higher speed, lower torque.

- Lead Angle: Compensates for winding inductance lag; improves torque and efficiency.

- Fault Codes & Troubleshooting

- When the status shows FAULT, refer to the list below:

- Hall Fault: Motor Hall sensor or wiring failure.

- Hardware Overcurrent: Abnormal hardware current; possible power-stage damage.

- Phase Overcurrent: Software-detected overcurrent; check power devices.

- Overvoltage: Input exceeds protection threshold; check settings or battery.

- Undervoltage: Input below protection threshold; low battery or incorrect settings.

- Throttle Fault: Abnormal throttle signal.

- Throttle Not Reset at Power-On: Safety interlock; reset throttle before power-on.

- Controller Overheating: High load; cool before reuse.

- Motor Overheating: Excessive load; cool before reuse.

- Stall Protection: Severe overload; reduce load.

- Phase Loss: Missing motor phase signal; check wiring.

- MOSFET Fault: Power transistor failure; repair or replace controller.

- Sensor Offset Fault: Internal current sensor error; restart or inspect controller.

- Motor Temperature Sensor Fault: Motor thermistor abnormality.

- Hardware Port & Advanced Functions

- Hardware Port Level Status: Updates with vehicle state (e.g., brake input switches port 2 to high).

- Start Self-Study: When a new motor is connected to the controller and the motor parameters are unknown, parameter settings must be configured via self-learning.

- After successful self-learning, the corresponding parameters can be viewed in the motor parameter interface.

- Data Record: Log operating data.

3.2 Protection (Voltage & Temperature Settings )

Click Write Parameters to save changes.

- Battery Voltage Settings

- Rated Voltage (V): Design operating voltage; base reference for all calculations.

- Overvoltage Protection (V): Cuts power if voltage is too high.

- Overvoltage Recovery (V): Voltage to resume operation after overvoltage.

- Undervoltage Protection (V): Cuts power to prevent deep discharge.

- Undervoltage Recovery (V): Voltage to resume after undervoltage.

- Low-Voltage Power Reduction (V): Smoothly limits power before undervoltage.

- Maximum Regenerative Voltage (V): Limits regeneration to avoid overcharging.

- Typical Voltage Settings

| Item | 48V | 60V | 72V | 84V | 96V |
| --- | --- | --- | --- | --- | --- |
| Rated Voltage | 48 | 60 | 72 | 84 | 96 |
| Overvoltage Protection | 60 | 75 | 90 | 105 | 120 |
| Overvoltage Recovery | 55 | 69 | 83 | 98 | 113 |
| Undervoltage Protection | 39 | 48 | 60 | 69 | 78 |
| Undervoltage Recovery | 43 | 52 | 64 | 73 | 82 |
| Low-Voltage Power Reduction | 44 | 53 | 65 | 74 | 83 |
| Max Regenerative Voltage | 57 | 71 | 86 | 101 | 116 |

- Basic Setting Rules

- Overvoltage Protection ≈ Rated Voltage × 1.25

- Overvoltage Recovery = Overvoltage Protection − 5–7 V

- Undervoltage Protection ≈ Rated Voltage × 0.83

- Undervoltage Recovery = Undervoltage Protection + 4–6 V

- Low-Voltage Power Reduction = Undervoltage Protection + ~5 V

- Max Regenerative Voltage = Overvoltage Protection − ~4 V

- Controller Temperature Protection

- Over-temperature Derating Point (°C): Begins gradual power reduction.

- Over-temperature Recovery Point (°C): Temperature to restore full power.

- Over-temperature Trip Point (°C): Shuts down completely to prevent burnout.

- Motor Temperature Protection

- Follows the same logic as controller thermal protection.

- 3.3 RUN (Speed & Current Parameters)

Click Write Parameters to save changes.

- Rated Speed (RPM)

- Low / Medium / High Gear Speed (RPM)

- S Gear Speed Limit (RPM)

- Min / Max Regenerative Speed (RPM)

- Reverse Speed (RPM)

- Current Settings

- Maximum Bus Current (A): Battery discharge limit.

- Maximum Phase Current (A): Motor phase current limit.

- Maximum Field-Weakening Current (A)

- Brake / Coasting Regeneration Current (A)

- Current Percentage per Gear

- Sets the fraction of maximum current available in each gear.Example: Max bus current = 100 A; low gear = 20% → 20 A.

3.4 AC/DC Current

FOC current-loop PI parameters (Iq/Kp/Ki, Id/Kp/Ki); use default values unless tuned.

3.5 P/N Torque (Forward & Reverse Torque)

Adjusts phase current levels and response across speed ranges to refine riding feel.

3.6 Function Settings

Click Write Parameters to save changes.

- Instrument & Coefficient Settings

- Adjusts speed/mileage display ratio for different meter types:

- Phase Line Meter

- Pulse (Hall) Meter

- Single-Wire Meter

- CAN Meter

- Throttle Parameters

- Throttle Lower Threshold (V): Default 1.0 V (range: 0.8–1.2 V)

- Throttle Midpoint (V): Default 2.5 V (range: 2.0–3.0 V)

- Throttle Upper Threshold (V): Default 4.0 V (range: 3.8–4.2 V)

- Throttle Midpoint Ratio (%): Default 50% (range: 30–70%)

- Gear, Acceleration & Anti-Rollback

- Set default gear and shift mode.

- Adjust forward/reverse acceleration/deceleration feel.

- Tune anti-rollback PI gains and speed threshold.

- Function Switches (Check to Enable)

- P Gear

- Forward

- Reverse

- High-Level Brake

- Low-Level Brake

- Anti-Theft Motor Lock

- Side Stand

- Auto-Calibration

- Brake Regeneration

- Coasting Regeneration

- TCS

- Cruise Control

- Anti-Rollback

- CANBUS

- Motor Temperature Protection

3.7 Motor Parameters

Number of Pole Pairs: Determines base motor speed.

Hall / Phase Connection: Set automatically by auto-calibration.

Forward / Reverse Angle Offset: Calibrated automatically; do not edit manually.

Auto-Calibration Current: Calibration current for motor angle detection.

Back EMF Coefficient: Measured during auto-calibration.

Version:The version name of the current software.